# URDF INAIL 2 ARM

BASE   (Link 0)

VOLUME =  7.3444716e+05  MM^3

SURFACE AREA =  3.6175122e+05  MM^2

AVERAGE DENSITY =  3.7803881e-06 KILOGRAM / MM^3

MASS =  2.7764953e+00 KILOGRAM

CENTER OF GRAVITY with respect to INAIL_BASE coordinate frame:

X   Y   Z     2.0548930e-01 -6.2095789e-01  7.7352002e+01  MM

INERTIA with respect to INAIL_BASE coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.6615891e+04  4.1822296e+01 -9.5386067e+01

Iyx Iyy Iyz  4.1822296e+01  2.7341339e+04  6.3543227e+00

Izx Izy Izz -9.5386067e+01  6.3543227e+00  6.2167916e+03

INERTIA at CENTER OF GRAVITY with respect to INAIL_BASE coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.0002127e+04  4.1468014e+01 -5.1253649e+01

Iyx Iyy Iyz  4.1468014e+01  1.0728527e+04 -1.2700723e+02

Izx Izy Izz -5.1253649e+01 -1.2700723e+02  6.2156037e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   6.2113708e+03  1.0000274e+04  1.0734613e+04

ROTATION MATRIX from INAIL_BASE orientation to PRINCIPAL AXES:

0.01321        0.99820        0.05850

0.02798       -0.05885        0.99787

0.99952       -0.01154       -0.02871

ROTATION ANGLES from INAIL_BASE orientation to PRINCIPAL AXES (degrees):

angles about x  y  z -91.648          3.354        -89.242

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.7298270e+01  6.0014679e+01  6.2179140e+01  MM

---------------------------------------------

Shoulder –YAW (Link1)

VOLUME =  1.5451915e+05  MM^3

SURFACE AREA =  1.5269891e+05  MM^2

AVERAGE DENSITY =  2.6568905e-06 KILOGRAM / MM^3

MASS =  4.1054046e-01 KILOGRAM

CENTER OF GRAVITY with respect to SHOULDER-YAW coordinate frame:

X   Y   Z     8.4798664e-02 -2.0782662e+01  3.2183555e+01  MM

INERTIA with respect to SHOULDER-YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.8974947e+03 -1.1538244e+00  1.2752762e-01

Iyx Iyy Iyz -1.1538244e+00  1.6556165e+03  5.9849636e+02

Izx Izy Izz  1.2752762e-01  5.9849636e+02  9.8100202e+02

INERTIA at CENTER OF GRAVITY with respect to SHOULDER-YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.2949444e+03 -1.8773371e+00  1.2479428e+00

Iyx Iyy Iyz -1.8773371e+00  1.2303835e+03  3.2390229e+02

Izx Izy Izz  1.2479428e+00  3.2390229e+02  8.0367882e+02

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   6.2916968e+02  1.2949401e+03  1.4048969e+03

ROTATION MATRIX from SHOULDER-YAW orientation to PRINCIPAL AXES:

-0.00299        0.99995       -0.00965

-0.47430        0.00708        0.88033

0.88036        0.00721        0.47425

ROTATION ANGLES from SHOULDER-YAW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z -61.688         -0.553        -90.171

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 3.9147669e+01  5.6162554e+01  5.8498435e+01  MM

---------------------------------------------

Shoulder –Pitch (Link2)

VOLUME =  9.2926976e+05  MM^3

SURFACE AREA =  8.4392290e+05  MM^2

AVERAGE DENSITY =  3.4960772e-06 KILOGRAM / MM^3

MASS =  3.2487988e+00 KILOGRAM

CENTER OF GRAVITY with respect to SHOULDER_PITCH coordinate frame:

X   Y   Z    -3.6360670e-02  1.6072685e+02 -6.2849027e+01  MM

INERTIA with respect to SHOULDER_PITCH coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.8713059e+05  1.1690170e+02 -1.2982417e+01

Iyx Iyy Iyz  1.1690170e+02  1.9390186e+04  2.9911001e+04

Izx Izy Izz -1.2982417e+01  2.9911001e+04  1.7220321e+05

INERTIA at CENTER OF GRAVITY with respect to SHOULDER_PITCH coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  9.0371217e+04  9.7915275e+01 -5.5581551e+00

Iyx Iyy Iyz  9.7915275e+01  6.5574254e+03 -2.9068254e+03

Izx Izy Izz -5.5581551e+00 -2.9068254e+03  8.8276594e+04

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   6.4540439e+03  8.8379821e+04  9.0371372e+04

ROTATION MATRIX from SHOULDER_PITCH orientation to PRINCIPAL AXES:

-0.00116        0.00453        0.99999

0.99937       -0.03550        0.00132

0.03550        0.99936       -0.00449

ROTATION ANGLES from SHOULDER_PITCH orientation to PRINCIPAL AXES (degrees):

angles about x  y  z-163.573         89.732       -104.393

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 4.4571223e+01  1.6493588e+02  1.6678386e+02  MM

---------------------------------------------

Elbow (Link3)

VOLUME =  1.6571767e+05  MM^3

SURFACE AREA =  1.4076162e+05  MM^2

AVERAGE DENSITY =  3.1656979e-06 KILOGRAM / MM^3

MASS =  5.2461209e-01 KILOGRAM

CENTER OF GRAVITY with respect to ELBOW coordinate frame:

X   Y   Z    -1.2974906e-02  6.4319293e+01 -3.4296150e+01  MM

INERTIA with respect to ELBOW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  4.3673408e+03  7.4078621e-01  1.8117200e-01

Iyx Iyy Iyz  7.4078621e-01  1.6195642e+03  1.5411161e+03

Izx Izy Izz  1.8117200e-01  1.5411161e+03  3.6702696e+03

INERTIA at CENTER OF GRAVITY with respect to ELBOW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.5799732e+03  3.0297811e-01  4.1461879e-01

Iyx Iyy Iyz  3.0297811e-01  1.0025018e+03  3.8387214e+02

Izx Izy Izz  4.1461879e-01  3.8387214e+02  1.4999643e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   7.9382178e+02  1.5799712e+03  1.7086463e+03

ROTATION MATRIX from ELBOW orientation to PRINCIPAL AXES:

-0.00009       -0.99999        0.00396

0.87857        0.00181        0.47760

-0.47761        0.00352        0.87857

ROTATION ANGLES from ELBOW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z -28.529          0.227         90.005

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 3.8899351e+01  5.4878905e+01  5.7069877e+01  MM

---------------------------------------------

Forearm-Yaw  (Link4)

VOLUME =  5.2899372e+05  MM^3

SURFACE AREA =  4.9964329e+05  MM^2

AVERAGE DENSITY =  3.7895619e-06 KILOGRAM / MM^3

MASS =  2.0046544e+00 KILOGRAM

CENTER OF GRAVITY with respect to FOREARM-YAW coordinate frame:

X   Y   Z    -6.4528753e-02 -4.6448857e+01  1.3357443e+02  MM

INERTIA with respect to FOREARM-YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  6.3418815e+04 -2.3245859e+01 -1.2506277e+01

Iyx Iyy Iyz -2.3245859e+01  5.5900542e+04  1.9408239e+04

Izx Izy Izz -1.2506277e+01  1.9408239e+04  9.5463233e+03

INERTIA at CENTER OF GRAVITY with respect to FOREARM-YAW coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  2.3326477e+04 -1.7237335e+01 -2.9785179e+01

Iyx Iyy Iyz -1.7237335e+01  2.0133230e+04  6.9706014e+03

Izx Izy Izz -2.9785179e+01  6.9706014e+03  5.2212805e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   2.4703316e+03  2.2882519e+04  2.3328137e+04

ROTATION MATRIX from FOREARM-YAW orientation to PRINCIPAL AXES:

0.00103        0.06063       -0.99816

-0.36709        0.92850        0.05602

0.93018        0.36636        0.02321

ROTATION ANGLES from FOREARM-YAW orientation to PRINCIPAL AXES (degrees):

angles about x  y  z -67.497        -86.524        -89.031

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 3.5104102e+01  1.0683958e+02  1.0787487e+02  MM

---------------------------------------------

Wrist-Pitch (Link5)

VOLUME =  2.9410855e+05  MM^3

SURFACE AREA =  3.2473590e+05  MM^2

AVERAGE DENSITY =  3.5423709e-06 KILOGRAM / MM^3

MASS =  1.0418416e+00 KILOGRAM

CENTER OF GRAVITY with respect to WRIST-PITCH coordinate frame:

X   Y   Z    -4.7450211e-02  1.7403048e+01  4.0478984e+01  MM

INERTIA with respect to WRIST-PITCH coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  3.8676725e+03  2.2219178e-01  1.8213696e+01

Iyx Iyy Iyz  2.2219178e-01  2.8185254e+03 -7.7155684e+02

Izx Izy Izz  1.8213696e+01 -7.7155684e+02  2.1572802e+03

INERTIA at CENTER OF GRAVITY with respect to WRIST-PITCH coordinate frame:  (KILOGRAM * MM^2)

INERTIA TENSOR:

Ixx Ixy Ixz  1.8450264e+03 -6.3813838e-01  1.6212593e+01

Iyx Iyy Iyz -6.3813838e-01  1.1114155e+03 -3.7623514e+01

Izx Izy Izz  1.6212593e+01 -3.7623514e+01  1.8417393e+03

PRINCIPAL MOMENTS OF INERTIA:  (KILOGRAM * MM^2)

I1  I2  I3   1.1094823e+03  1.8281114e+03  1.8605876e+03

ROTATION MATRIX from WRIST-PITCH orientation to PRINCIPAL AXES:

-0.00026       -0.69221        0.72170

0.99868       -0.03722       -0.03533

0.05132        0.72074        0.69131

ROTATION ANGLES from WRIST-PITCH orientation to PRINCIPAL AXES (degrees):

angles about x  y  z   2.926         46.195         90.022

RADII OF GYRATION with respect to PRINCIPAL AXES:

R1  R2  R3 3.2633176e+01  4.1889047e+01  4.2259486e+01  MM

---------------------------------------------

Wrist-Roll (Link6)